Proposed Title: Deep Learning Based Defect Detection and Rule-Based Correction Framework for FDM 3D Printing

1. Topic:

This project investigates six FDM 3D printing defects using labeled Roboflow images, transfer-learned convolutional neural networks, and a rule-based correction recommendation framework.

2. Problem:

Fused Deposition Modeling (FDM) is an additive manufacturing technique that produces three-dimensional objects by depositing thermoplastic material layer by layer. Despite its accessibility and cost-effectiveness, the process is susceptible to defects such as stringing, warping, layer shifting, under-extrusion, over-extrusion, and poor bed adhesion. These defects can compromise dimensional accuracy, mechanical performance, surface quality, and material efficiency. Identifying printing defects and determining suitable corrective actions often require continuous human observation, technical expertise, and repeated adjustments to printing parameters, increasing production time and material waste. Previous research has demonstrated that deep learning and computer vision can identify printing abnormalities and support automated correction through feedback mechanisms. However, many existing implementations rely on specialized monitoring equipment, extensive training datasets, and real-time integration with physical printers, making them challenging to implement within limited computational resources and project timelines. Furthermore, the comparative performance of accessible transfer-learning architectures for classifying multiple FDM defects and their integration with interpretable corrective recommendations warrants further evaluation. A practical alternative is to combine image-based defect classification with a rule-based recommendation framework that links identified defects to potential corrective actions without requiring physical printer control. Therefore, this project develops and evaluates a framework using pretrained convolutional neural networks to classify six common FDM printing defects and a rule-based system to recommend corresponding adjustments to printing parameters.

3. Research Questions

RQ1. How accurately can a transfer-learned convolutional neural network classify six common FDM 3D printing defect types from images?

Method: Fine-tune a pretrained CNN on labeled images of stringing, warping, layer shifting, under-extrusion, over-extrusion, and bed adhesion failures. Evaluate classification performance on a held-out test set using accuracy, macro-averaged precision, recall, and F1-score.

RQ2. How does the classification performance of MobileNet compare with ResNet when trained on the same FDM printing defect dataset?

Method: Train MobileNet and ResNet using identical training, validation, and testing partitions and comparable training configurations. Compare their overall accuracy, macro-F1 score, per-class recall, and average inference time per image.

RQ3. How accurately can a rule-based correction framework associate the six classified FDM defects with appropriate corrective printing actions?

Method: Develop a defect-to-correction rule table using published research and established FDM troubleshooting principles. Evaluate the recommendations against independently prepared, literature-supported reference recommendations for predefined defect scenarios, using recommendation agreement rate.

4. Data

Dataset: 3D Printing Defect Classification Dataset

Dataset Source: Roboflow Universe

Dataset Link: https://universe.roboflow.com/project-jkfnh/3d-printing-defect-classification

Dataset Size: 22,048 images in the complete published dataset.

Dataset Description: The complete dataset contains 22,048 images organized into 19 listed categories, including different types of printing defects, a No Defect category, and an Unlabeled category.

For this research, six defect categories will be selected. The selected defects are:

- Stringing: Unwanted thin strands of filament appearing between printed features

- Warping: Deformation or lifting of printed regions due to shrinkage and thermal stresses

- Layer Shifting: Misalignment between successive printed layers

- Under-Extrusion: Insufficient filament deposition during printing

- Over-Extrusion: Excessive filament deposition during printing

- Bed Adhesion: Problems involving insufficient attachment of the printed object to the build surface

Labels and Targets:

The dataset provides image-level class labels corresponding to printing defect categories. These labels will serve as the ground-truth targets for supervised multiclass image classification.

The six selected classes will be encoded as numerical labels and used to train the CNN models to distinguish different types of printing defects.

Proposed Data Split:

- Training: 70%

- Validation: 15%

- Testing: 15%

A stratified splitting approach will be used to preserve the relative distribution of the six defect categories in each subset.

Data Access Status: The dataset is publicly listed on Roboflow Universe under the CC BY 4.0 license.

5. Related Work

1. Autonomous in-situ correction of fused deposition modeling printers using computer vision and deep learning.

- Jin, Zhang, and Gu (2019)

It developed a real-time monitoring and autonomous correction system using a pretrained ResNet-50 convolutional neural network to classify FDM printing conditions as good-quality, under-extrusion, or over-extrusion. Their model achieved over 98% classification accuracy, and the system demonstrated automated adjustment of filament flow rate to improve printing conditions. This research supports our use of transfer learning for defect classification and provides a foundation for connecting identified extrusion defects with appropriate corrective printing parameters.

2. Automated recognition and correction of warp deformation in extrusion additive manufacturing
- Brion, Shen, and Pattinson (2022)

It developed a framework combining deep learning, computer vision, and expert-informed heuristics to recognize warping defects and guide corrective printing adjustments. Their YOLOv3-based detector achieved a best-run validation mean average precision of 88.72%, and their experiments demonstrated that adjustments to printing conditions could reduce warping. Our project builds on the principle of combining learned defect recognition with interpretable correction rules, while extending the classification task to multiple defect categories and implementing recommendations without direct printer control.

3. Generalisable 3D printing error detection and correction via multi-head neural networks

- Brion and Pattinson (2022)

They developed a multi-head residual attention network trained on a large dataset of extrusion printing images to identify whether printing parameters were too low, acceptable, or too high. Their system achieved an overall test accuracy of 84.3% and demonstrated real-time correction across different printing conditions. This research highlights the potential of connecting image-based monitoring with manufacturing parameter adjustments, while our project investigates a simpler approach using transfer-learned CNN classifiers and predefined correction rules.

4. Deep learning-assisted real-time defect detection and closed-loop adjustment for additive manufacturing of continuous fiber-reinforced polymer composites

- Lu et al. (2023)

It investigated deep learning-assisted defect detection and closed-loop correction in robotic additive manufacturing of continuous fiber-reinforced polymer composites. Their comparison of Faster R-CNN, SSD, and YOLOv4 demonstrated that YOLOv4 provided the strongest overall detection performance, with a reported mean average precision of 90.42%, and supported the feasibility of adjusting manufacturing parameters based on recognized defects. The study supports the broader approach of integrating automated visual inspection with structured correction strategies.

5. Defect Detection and Closed-loop Feedback Using Machine Learning for Fused Filament Fabrication 
- De La Rosa, Armani, and Golmohamadi (2024)

They developed a defect detection and feedback system using transfer learning with VGG-16 to identify printing abnormalities, including stringing, under-extrusion, and over-extrusion. Their CNN achieved 92.86% validation accuracy and 90.0% testing accuracy, while the correction framework incorporated adjustments to feed rate, nozzle temperature, extrusion amount, and cooling fan speed. This study closely relates to our proposed framework because it demonstrates how classification results can be connected to printing-parameter adjustments, which our project will implement through a simulated rule-based recommendation system.

Positioning of the Proposed Research

Previous studies have demonstrated that deep learning can effectively identify 3D printing defects and that defect-informed adjustment of manufacturing parameters can improve printing quality. However, many existing correction frameworks require extensive datasets, dedicated monitoring hardware, or real-time printer integration. Our proposed project investigates a computationally accessible alternative by combining multiclass defect classification using transfer-learned MobileNet and ResNet models with an interpretable rule-based correction recommendation system. Using a selected subset of the Roboflow dataset, the project will evaluate the classification of six common FDM printing defects and the ability to associate predicted defect categories with appropriate corrective suggestions. Rather than implementing physical closed-loop control, this research will provide an experimental proof of concept for integrating deep learning-based defect recognition with manufacturing decision support.